Binding

Machine

Instructions

If after reading these instructions you need further guidance,

please ask a member of library staff.

HOW IT WORKS

Binding comb

Ask staff at the issue desk for a binding comb and

clear plastic/card cover sheets (50p per set)

QUICK GUIDE

- Place comb in machine, open comb.

- Punch holes in sheets, in bunches of not more than           10 pages or 2 plastic sheets.

- Load sheets onto comb in batches as they are punched.

- Release comb to finish binding.

step-by-step guide on the following pages

Trouble-shooting guide at end of manual

LEVER

LID

INSERTING THE BINDING COMB

HANDLE

TEETH

TEETH

Lift the lid and handle.

HANDLE

Check lever to side is fully pushed back.

- Place the binding comb behind the row of silver teeth

with curls opening up toward the top; comb rings

FRONT OF THE MACHINE

should go between the teeth.

- Pull lever gently towards you to open

the comb (do not pull it too far!)

LEVER

VIEW FROM BEHIND TEETH

WITH HANDLE LIFTED

PUNCHING THE HOLES

1.    Only punch 10 sheets of paper or 2 plastic sheets at a time

– more will cause a jam!

2.    Lift handle.

3.    Insert sheets into slot towards the back of the machine

(alongside the black dotted line, which indicates where the

hole punches will be). Make sure the papers are as far in

and as far to the left as possible.

HANDLE

4.    Pull down handle.

PAPER

5.    Return handle to upright

position to release pages.

6.    Remove pages.

BINDING THE SHEETS TOGETHER

- Place the batches of paper on the comb as they are punched to keep them in order and keep all the holes aligned

(if applicable, remember to start and end with your cover  sheets).

- Once all sheets are placed in the comb push back lever to original position to release completed document.

PAPER

LEVER

HAVING TROUBLE

Binding comb doesn’t open:

FRONT OF THE MACHINE

Try turning the binding comb over so that the curls open up toward the top and the comb is behind the teeth. Make sure the lever to the side of the machine is completely pushed back before inserting comb.

CURLS OPEN UP TOWARD THE TOP

COMB IS BEHIND THE TEETH

VIEW FROM BEHIND TEETH

Crooked punch-hole line:

Try placing the papers in the entry as evenly as possible and pushing them in and to the left as far as possible. Otherwise ask library staff to clean out the paper scraps that fall into a tray at the bottom of the machine (see next page).

Stuck handle / paper jam:

DO NOT tear out the paper. Push down on the handle as hard as possible and as many times as necessary to clear the jam. Try punching smaller batches of paper.

Clearing out the paper entry:

Take a ruler or stiff cardboard and slide it into the paper entry, moving it back and forth to release paper cuttings into the waste drawer. Empty the waste drawer, which is accessed via the right hand side of the machine.

The last hole goes off the edge of the paper:

Make sure you line up the sheets of paper with the purple edge guide. This can be set to either A4 or A5 by rotating the purple dial. The black broken line on the machine indicates the position of the hole punches.

If you continue to have problems please come to the

issue desk to ask a member of staff for help